Four-Faith Zigbee Firmware Upgrade Guide

The Zigbee upgrade steps:

1.Connect the Zigbee and PC with the shipped RS-232 or RS-485 cable , Don’t power the Zigbee.

- Run the upgrade tool

3. Press the “Open” button to open the serial port, press “Load” button to select the Zigbee upgrade firmware ,Press “DOWN” button for the coming upgrade process.

4.Power the Zigbee and begin upgrading.

- When“Download OK Upgrade Finished！！” prompted ,the upgrade process finished !